VAULTSCOPE

AI-Powered IPsec VPN Protocol Analyzer & Security Assessment Framework

────────────────────────────────────────

Full Product Specification & Engineering Reference

SIH 2026 · Problem Statement SIH26160 · NTRO

Prepared for async team development across 4 engineers

# TABLE OF CONTENTS

1.  Problem Statement & Scope	3

2.  What We Are Building	4

3.  Architecture Overview	5

4.  Component Design (All Stages)	6

5.  Data Models	12

6.  Tech Stack	14

7.  SOTA Comparison	15

8.  Task Assignment — 4 Engineers	16

9.  Commit Map & Test Cases	20

10.  Dataset Specification	28

11.  API Contract	29

12.  Risk Register	31

13.  Deliverables Checklist	32

# 1. PROBLEM STATEMENT & SCOPE

## 1.1 What NTRO Wants

NTRO (National Technical Research Organisation) manages large-scale VPN infrastructure across sensitive government networks. IPsec VPNs are only as secure as the cryptographic parameters negotiated between peers — but in practice, enterprise IPsec deployments accumulate years of configuration drift. Old devices running IKEv1 with DES and DH group 2 sit next to modern IKEv2 deployments. Administrators have no continuous visibility into what is actually being negotiated on the wire versus what the config file says.

The PS title says "AI-Powered" — the AI requirement is real and will be evaluated. However the primary deliverable is the protocol analysis and security assessment engine. The AI sits on top as anomaly detection, traffic classification, and risk scoring intelligence.

## 1.2 Two Distinct AI Problems

Reading the PS carefully, there are two separate AI/ML problems of very different difficulty:

- Protocol/crypto-parameter identification — mostly deterministic. IKE negotiation payloads declare cipher suites, DH groups, and auth methods in cleartext during handshake. This is closer to a parser + classifier hybrid. Not hard ML.

- Traffic-type-inside-ESP prediction — genuinely hard. Since the ESP payload is encrypted, we can only infer what application traffic (VoIP, video, web, email, ICMP) is running inside the tunnel from metadata: packet size distribution, inter-arrival timing, directionality ratio, and burst patterns. This is the actual ML showpiece.

## 1.3 Explicit Deliverables (from PS)

The PS calls out these artifacts explicitly — all must be present at submission:

| Deliverable | Description | Owner |
| --- | --- | --- |
| Prototype / working tool | Functional end-to-end system (pcap upload → report) | All |
| AI Engine | Trained traffic-type classifier + protocol classifier | P2 |
| Dashboard | React frontend with session table, graph, drilldown | P4 |
| Dataset | Labeled pcap files + ground-truth JSON metadata | P1 |
| Sample Report | Executive PDF + Technical HTML from a demo capture | P3 |
| Demo Video | 3–5 min walkthrough of the working system | All |
| Documentation | This document + API reference + setup guide | All |

# 2. WHAT WE ARE BUILDING

VaultScope is a passive-first, active-capable IPsec VPN security intelligence platform. It ingests traffic at the packet level, reconstructs the complete cryptographic negotiation state of every VPN session, scores each session against a living vulnerability database, classifies the traffic type running inside each encrypted tunnel, models attack-surface exposure as a peer graph, and produces layered reports from raw session data to an executive posture score — with exact, vendor-specific remediation config diffs.

## 2.1 What It Is NOT

- Not a scanner — does not send packets to discover peers (active mode is secondary, not primary)

- Not a wrapper around ike-scan or ikeforce — those are pentest tools; VaultScope is a continuous assessment platform

- Not a firewall policy tool — Tufin/AlgoSec analyse config files; VaultScope analyses wire traffic

- Not a black-box AI — every finding is linked to a specific RFC, CVE, or NIST standard

## 2.2 Modes of Operation

| Mode | Input | Use Case | Packets Sent? |
| --- | --- | --- | --- |
| Passive (primary) | pcap file upload or live NIC mirror | Continuous monitoring of own infra | None — invisible |
| Active (secondary) | Target IP list | Point-in-time assessment of peer endpoints | Yes — IKE probes only |

# 3. ARCHITECTURE OVERVIEW

## 3.1 Pipeline Diagram

┌─────────────────────────────────────────────────────────────────────┐

│  STAGE 0: TESTBED (offline, pre-hack)                               │

│  Docker/netns pairs: strongSwan configs × cipher × DH × traffic     │

│  Output: labeled pcap files + ground-truth JSON (the dataset)       │

└──────────────────────────────┬──────────────────────────────────────┘

│ pcap + labels

▼

┌─────────────────────────────────────────────────────────────────────┐

│  STAGE 1: INGESTION ENGINE                                          │

│  pyshark/scapy → normalize → session bucket by (SPI_i, SPI_r)      │

└──────────────────────────────┬──────────────────────────────────────┘

▼

┌──────────────────────┴──────────────────────┐

▼                                             ▼

┌───────────────────────┐               ┌────────────────────────────┐

│  STAGE 2: IKE PARSER  │               │  STAGE 3: ESP FLOW         │

│  IKEv1/v2 handshake   │               │  FEATURE EXTRACTOR         │

│  → VPNSession object  │               │  packet size, IAT, burst   │

└──────────┬────────────┘               └──────────┬─────────────────┘

│                                        │

┌─────┴──────────────────────────────────┬───┘

▼                                        ▼

┌──────────────────┐  ┌──────────────────┐  ┌──────────────────────┐

│ STAGE 4a:        │  │ STAGE 4b:         │  │ STAGE 4c:            │

│ Protocol/Crypto  │  │ Traffic-Type      │  │ Security Rule Engine │

│ Classifier       │  │ Classifier (ML)   │  │ CVE-linked scoring   │

└────────┬─────────┘  └────────┬──────────┘  └──────────┬───────────┘

└────────────────┬─────┘──────────────────────┘

▼

┌─────────────────────────────────────────────────────────────────────┐

│  STAGE 5: SCORING & REPORT AGGREGATOR                              │

│  Risk Score + Threat Matrix + AI Confidence → PDF/HTML/JSON/CEF    │

└──────────────────────────────┬──────────────────────────────────────┘

▼

┌─────────────────────────────────────────────────────────────────────┐

│  STAGE 6: DASHBOARD / UI                                           │

│  React: upload, live view, session table, graph, export            │

└─────────────────────────────────────────────────────────────────────┘

## 3.2 Repo Structure

vaultscope/

├── testbed/          # Stage 0: configs, traffic generators, pcap capture

├── core/

│   ├── ingestion/    # Stage 1: pyshark wrapper, session bucketing

│   ├── ike_parser/   # Stage 2: IKEv1/v2 state machine + payload decoders

│   ├── flow/         # Stage 3: ESP flow feature extractor

│   ├── classifiers/  # Stage 4a + 4b: protocol + traffic-type ML

│   └── rules/        # Stage 4c: security rule engine + CipherDB

├── reporting/        # Stage 5: scoring aggregator + Jinja2 report templates

├── api/              # FastAPI backend + WebSocket

├── frontend/         # Stage 6: React + Recharts + D3

├── data/             # Dataset: pcaps + ground-truth JSONs

├── tests/            # All test cases (mirrored structure of core/)

└── docs/             # This document + API reference + setup guide

# 4. COMPONENT DESIGN

## Stage 0 — Testbed & Dataset Generator

### Purpose

Build a controlled lab environment that produces labeled IPsec traffic for both ML training and system testing. This is a deliverable in itself — the dataset is an explicit PS output.

### Tooling

- strongSwan — primary VPN daemon, best cipher-suite flexibility via swanctl.conf

- Docker Compose + Linux network namespaces (netns) — lightweight isolated peer pairs, no full VMs needed

- sipp — VoIP (SIP/RTP) traffic generation

- ffmpeg — video streaming over RTP/HTTP

- curl / headless Chromium — web traffic simulation

- Postfix + swaks — email (SMTP) traffic

- ping / hping3 — ICMP traffic

- Custom Python burst generator — approximates WhatsApp-like bursty encrypted chat traffic (NOTE: literal WhatsApp cannot run in isolated lab; substitute disclosed in reports)

### Config Matrix

Script generates strongSwan configs by Cartesian product of:

| Dimension | Values |
| --- | --- |
| IPsec Mode | tunnel, transport |
| Encryption | AES-128-CBC, AES-256-CBC, AES-128-GCM, AES-256-GCM, 3DES-CBC (weak), DES-CBC (weak) |
| Integrity / PRF | HMAC-SHA1 (weak), HMAC-SHA256, HMAC-SHA512, implicit (AEAD) |
| DH Group | MODP-1024 (group 2, weak), MODP-2048 (group 14), ECP-256 (group 19), ECP-521 (group 21) |
| PFS | enabled, disabled |
| IP Version | IPv4, IPv6 |
| Traffic Type | VoIP, Video, Web, Email, ICMP, Chat (WhatsApp proxy) |

### Dataset Output Schema

data/

├── pcaps/

│   └── tunnel_aes256gcm_ecp521_pfs_ipv4_voip_001.pcap

└── labels/

└── tunnel_aes256gcm_ecp521_pfs_ipv4_voip_001.json

{

"filename": "tunnel_aes256gcm_ecp521_pfs_ipv4_voip_001.pcap",

"ipsec_mode": "tunnel",

"ike_version": "IKEv2",

"encryption": "AES-256-GCM",

"integrity": "implicit",

"dh_group": "ECP521",

"pfs_enabled": true,

"ip_version": "IPv4",

"traffic_type": "VoIP",

"capture_duration_sec": 120,

"known_weak": false

}

## Stage 1 — Ingestion Engine

### Purpose

Accept a pcap file or live NIC interface and produce per-session packet streams bucketed by IKE session identifiers.

### Implementation

- Primary: pyshark (tshark wrapper) — structured field access, handles malformed packets more gracefully than raw scapy

- Fallback: scapy — for raw parsing when pyshark field extraction misses vendor-proprietary payloads

- Filters: UDP port 500 (IKE), UDP port 4500 (NAT-T), IP protocol 50 (ESP), IP protocol 51 (AH)

- Session bucketing key: (SPI_initiator, SPI_responder) for IKEv2; (cookie_i, cookie_r) for IKEv1

- Handles mid-session captures — sessions without observed IKE handshake are flagged incomplete

### Output

Per-session RawSession objects passed to Stage 2 (IKE stream) and Stage 3 (ESP stream) in parallel.

## Stage 2 — IKE Parser

### Purpose

Reconstruct the full IKE handshake from raw packet stream and extract every negotiated cryptographic parameter.

### IKEv2 Message Handling

- IKE_SA_INIT (messages 1-2): extract SA proposals → accepted ENCR/PRF/INTEG/DH transforms

- IKE_AUTH (messages 3-4): extract AUTH method, ID payload, CERT if present

- CREATE_CHILD_SA: detect KE payload → PFS enabled; absence of KE → PFS disabled

- Transport vs Tunnel: USE_TRANSPORT_MODE Notify payload in IKE_AUTH → transport mode

### IKEv1 Message Handling

- Main Mode (6 messages): SA → KE → ID (encrypted) — identity protected

- Aggressive Mode (3 messages): SA + KE + ID in first message — CRITICAL: ID payload in message 1 is unencrypted, PSK hash exposed and offline-crackable. Detection signal: presence of ID payload in first IKE message, not message count (retransmissions can alter count)

- Quick Mode: detect KE payload → PFS; extract Phase 2 cipher suite

### Edge Cases

- Mid-session capture (no IKE handshake observed): mark pfs_status = "unknown" NOT "disabled" — prevents false-positive risk penalty

- Fragmented IKE (RFC 7383): reassemble before parsing — also flag as anomaly on Cisco ASA (CVE-2016-1287 vector)

- Vendor ID payloads: hash-match against fingerprint DB → infer implementation (strongSwan, Cisco ASA, Juniper SRX, Fortinet, Palo Alto, OpenSwan)

### Output — VPNSession Object

{

"session_id": "spi_i-spi_r",

"ike_version": "IKEv1 | IKEv2",

"mode": "tunnel | transport",

"aggressive_mode": false,

"encryption": "AES-256-GCM",

"integrity": "implicit (AEAD)",

"prf": "PRF_HMAC_SHA2_256",

"dh_group": "ECP256",

"pfs_status": "enabled | disabled | unknown",

"auth_method": "PSK | RSA | EAP | XAUTH",

"ip_version": "IPv4 | IPv6",

"sa_lifetime_sec": 3600,

"vendor": "strongSwan | Cisco ASA | Juniper SRX | unknown",

"nat_traversal": false,

"fragmented_ike": false,

"capture_complete": true

}

## Stage 3 — ESP/AH Flow Feature Extractor

### Purpose

Since ESP payload is encrypted, infer traffic characteristics from metadata only. Produces a feature vector per flow for the Stage 4b traffic-type classifier.

### Feature Vector (per flow)

| Feature | Description | Why It Matters |
| --- | --- | --- |
| pkt_size_mean | Mean ESP packet size (bytes) | VoIP: ~180B constant; Video: ~1400B large; ICMP: tiny fixed |
| pkt_size_std | Std deviation of packet sizes | VoIP: near-zero; Web: high variance |
| pkt_size_p10/p90 | 10th and 90th percentile | Robust outlier handling |
| iat_mean | Mean inter-arrival time (ms) | VoIP: very regular; Web: irregular |
| iat_std | Std dev of inter-arrival time | VoIP: near-zero; ICMP: variable |
| dir_ratio | Bytes up / bytes down | Email: high up; Web: high down; VoIP: ~1.0 |
| burst_count | Number of burst events (>10 pkts in 100ms) | Video: many large bursts |
| burst_gap_ratio | Avg burst duration / avg gap between bursts | Distinguishes Video (long bursts) from Web (short bursts) |
| payload_size_var_burst | Variance of pkt size WITHIN bursts | Video/Web vs VoIP separation |
| flow_duration_sec | Total session duration | VoIP: long; Web: short; Email: medium |
| pkt_total | Total packet count | Normalizes counts |
| rate_pps | Packets per second | VoIP: constant; Video: variable |

### Implementation

- Primary: custom scapy-based flow aggregator (no C extension dependencies — safer than nfstream in hackathon environment)

- nfstream as optional accelerator if available — test Docker image pre-hack

## Stage 4a — Protocol/Crypto Classifier

### Purpose

Classify protocol and cryptographic parameters for sessions where deterministic IKE parsing is incomplete (truncated captures, vendor extensions, malformed payloads).

### Implementation

- Primary path: deterministic parser output from Stage 2 — no model needed

- Fallback path (ambiguous cases only): small Random Forest over partial IKE fields + IKE message size patterns (cipher suites produce measurably different IKE message sizes due to padding)

- Output: same VPNSession schema as Stage 2, with confidence_source = "parser" or "classifier"

## Stage 4b — Traffic-Type Classifier (Core ML Component)

### Purpose

Given the ESP flow feature vector from Stage 3, predict the traffic type running inside the encrypted tunnel. This is the primary AI showpiece of the product.

### Model

- Baseline: RandomForestClassifier (scikit-learn) — fast, interpretable, works well on small labeled datasets, produces feature importances for judge demo

- Stretch: XGBoost — better on structured tabular features with moderate dataset size

- Stretch (if time): 1D-CNN over raw packet-size-sequence — VoIP and Video have highly distinctive temporal signatures that benefit from sequence modeling

### Training

- Data: feature vectors extracted from Stage 0 testbed pcaps — label = injected traffic type

- Split: 70% train / 15% validation / 15% test — stratified by traffic type and cipher config

- Expected separation: VoIP, ICMP easy; Video vs Web hardest — use burst_gap_ratio and payload_size_var_burst as key discriminators

- Output per session: predicted_type + confidence (softmax probability) → feeds AI Confidence Score deliverable

### Evaluation Artifacts for Judges

- Confusion matrix (all 6 classes)

- Per-class precision/recall/F1

- Feature importance bar chart (Random Forest)

- Honest disclosure: lab-controlled training data; real-world accuracy will be lower due to network noise — state explicitly in technical report

## Stage 4c — Security Rule Engine

### Purpose

Evaluate each VPNSession against a structured rule table to identify security weaknesses, assign severity, link to CVEs and standards.

### Rule Table

| Rule ID | Check | Weak Condition | Severity | Reference |
| --- | --- | --- | --- | --- |
| R01 | Encryption algorithm | DES, NULL | CRITICAL | NIST SP 800-77, RFC 8247 |
| R02 | Encryption algorithm | 3DES-CBC (SWEET32 threshold) | HIGH | CVE-2016-2183, RFC 8247 |
| R03 | DH Group | Group 1 (768-bit MODP) | CRITICAL | RFC 8247 §4, LOGJAM |
| R04 | DH Group | Group 2 (1024-bit MODP) | CRITICAL | RFC 8247 §4, LOGJAM |
| R05 | DH Group | Group 5 (1536-bit MODP) | HIGH | RFC 8247 §4 |
| R06 | IKE Mode | IKEv1 Aggressive Mode + PSK | CRITICAL | PSK hash exposed in cleartext |
| R07 | IKE Version | IKEv1 any mode | HIGH | RFC 9395 (IKEv1 deprecated) |
| R08 | Integrity | MD5-HMAC | CRITICAL | RFC 8247, SLOTH attack |
| R09 | Integrity | SHA1-HMAC | HIGH | RFC 8247 §3 |
| R10 | PFS | Disabled | MEDIUM | Best practice, CNSSP-15 |
| R11 | SA Lifetime | >86400 sec (24h) | MEDIUM | NIST SP 800-77 §5 |
| R12 | SA Lifetime | >28800 sec (8h) | LOW | Operational hygiene |
| R13 | Auth method | PSK with any weak DH (R03-R05) | HIGH | Offline PSK crack risk |
| R14 | IKE fragmentation | Fragmented IKE on Cisco ASA | HIGH | CVE-2016-1287 |
| R15 | Replay protection | Anti-replay disabled (AH/ESP seq=0) | HIGH | RFC 4303 §3.4.3 |

### Scoring Model

- Each triggered rule contributes a weighted penalty (CRITICAL=40, HIGH=20, MEDIUM=10, LOW=5) to a raw penalty score

- Composite risk score = max(0, 100 - raw_penalty_sum), clamped

- CRITICAL rules auto-set overall severity to CRITICAL regardless of composite score

## Stage 5 — Scoring & Report Aggregator

### Purpose

Combine Stage 4a+4b+4c outputs per session into a unified assessment record, generate all report artifacts.

### Outputs

| Output | Format | Audience | Contents |
| --- | --- | --- | --- |
| Executive Report | PDF (1-2 pages) | CISO / DG level | Overall posture score, top 3 findings, plain-English remediation |
| Technical Report | HTML + PDF | Security engineers | Full session table, per-finding CVE links, vendor config diffs, ML confidence |
| Machine Export | JSON + CEF | SIEM ingestion | Complete VPNSession array, CEF-formatted for direct SIEM import |
| Dataset Export | ZIP (pcaps + JSONs) | PS deliverable | All labeled training data |

### Vendor-Specific Remediation Engine

Every finding generates an exact config diff for the detected vendor implementation:

- Cisco ASA / FTD: crypto ikev2 policy syntax

- strongSwan: swanctl.conf proposals= format

- Juniper SRX: set security ike proposal syntax

- Fortinet FortiGate: config vpn ipsec phase1-interface syntax

- Palo Alto Networks: IKE gateway crypto profile

- Unknown vendor: RFC-compliant generic recommendation

## Stage 6 — Dashboard / UI

### Purpose

React single-page application with two primary views: live capture and analysis.

### Views

- Upload / live-capture selector — pcap file drop or live NIC selector

- Session table — sortable/filterable by risk score, protocol version, cipher, peer IP, anomaly flags

- Peer graph — D3.js force-directed, node colour = worst session score, edges = sessions, click to drilldown

- Session drilldown — full IKE decode, rule violations, classifier confidence, vendor config diff

- Aggregate dashboard — risk distribution histogram, traffic-type pie chart, threat matrix heatmap

- Historical comparison — load two pcaps, diff sessions (new peers, degraded scores, cipher changes)

- Export panel — executive PDF, technical HTML, JSON/CEF download

# 5. DATA MODELS

## 5.1 Core Session Record (DB schema)

{

"session_id":       "string  — (spi_i)-(spi_r) or (cookie_i)-(cookie_r)",

"capture_source":   "string  — pcap_upload | live_nic | active_probe",

"capture_file":     "string  — filename if pcap upload",

"timestamp":        "string  — ISO8601 of first IKE packet",

"initiator_ip":     "string",

"responder_ip":     "string",

"ike": {

"version":         "IKEv1 | IKEv2",

"mode":            "tunnel | transport",

"aggressive_mode": "bool",

"encryption":      "string  — e.g. AES-256-GCM",

"integrity":       "string  — e.g. HMAC-SHA256 | implicit",

"prf":             "string  — e.g. PRF_HMAC_SHA2_256",

"dh_group":        "string  — e.g. ECP256 | MODP1024",

"pfs_status":      "enabled | disabled | unknown",

"auth_method":     "PSK | RSA | EAP | XAUTH",

"ip_version":      "IPv4 | IPv6",

"sa_lifetime_sec": "int",

"vendor":          "string  — fingerprinted impl or unknown",

"nat_traversal":   "bool",

"fragmented_ike":  "bool",

"capture_complete":"bool"

},

"flow_features": {

"pkt_size_mean":   "float",

"pkt_size_std":    "float",

"pkt_size_p10":    "float",

"pkt_size_p90":    "float",

"iat_mean_ms":     "float",

"iat_std_ms":      "float",

"dir_ratio":       "float  — bytes_up / bytes_down",

"burst_count":     "int",

"burst_gap_ratio": "float",

"payload_size_var_burst": "float",

"flow_duration_sec":"float",

"pkt_total":       "int",

"rate_pps":        "float"

},

"traffic_prediction": {

"predicted_type":  "VoIP | Video | Web | Email | ICMP | Chat | Other",

"confidence":      "float  — softmax probability 0.0-1.0",

"model_version":   "string"

},

"security_assessment": {

"risk_score":      "int    — 0-100 (100=safe)",

"overall_severity":"CRITICAL | HIGH | MEDIUM | LOW | SAFE",

"triggered_rules": ["R01", "R06", ...],

"findings": [

{

"rule_id":    "string",

"description":"string",

"severity":   "string",

"cve":        "string | null",

"standard":   "string",

"remediation":"string  — vendor-specific config diff"

}

],

"threat_matrix":  [{"threat":"string","likelihood":"Low|Med|High","impact":"Low|Med|High"}],

"ai_confidence":  "float  — aggregate classifier confidence"

},

"reports": {

"executive_pdf":  "string | null  — file path",

"technical_html": "string | null",

"json_export":    "string | null",

"cef_export":     "string | null"

}

}

## 5.2 Anomaly Event Record

{

"anomaly_id":     "uuid",

"session_id":     "string",

"timestamp":      "ISO8601",

"anomaly_type":   "string  — e.g. AGGRESSIVE_MODE_PROBE | SPI_REPLAY | TRANSFORM_BRUTEFORCE",

"severity":       "CRITICAL | HIGH | MEDIUM",

"description":    "string",

"evidence_pkts":  [int]   — packet numbers in source pcap

}

# 6. TECH STACK

| Layer | Technology | Reason |
| --- | --- | --- |
| VPN testbed | strongSwan, Docker Compose, Linux netns | Best cipher-suite flexibility; netns lightweight vs full VMs |
| Traffic generation | sipp (VoIP), ffmpeg (video), curl (web), Postfix+swaks (email), hping3 (ICMP) | One dedicated tool per traffic class |
| Packet capture | tshark, tcpdump, pyshark, scapy | pyshark primary; scapy fallback for raw parsing |
| Flow features | Custom scapy aggregator (nfstream optional) | Avoids C extension build issues in Docker; nfstream as accelerator if available |
| IKE parsing | pyshark + custom payload decoders | RFC 7296 (IKEv2) + RFC 2409 (IKEv1) structures |
| ML — protocol classifier | scikit-learn DecisionTree / RandomForest | Handles ambiguous/partial captures |
| ML — traffic classifier | scikit-learn RandomForest + XGBoost baseline; PyTorch 1D-CNN stretch | RF: fast and interpretable; CNN stretch: better temporal modelling |
| Rule engine | Python, rules as YAML | Auditable, easy to update CipherDB and rule weights |
| Report generation | Jinja2 templates + WeasyPrint (PDF) | No external service dependency |
| Backend / API | FastAPI + WebSocket | Async; WebSocket for live session stream to UI |
| Database | SQLite (single-node, no concurrent write contention in assessment workflow) | Swap to PostgreSQL for production; deliberate choice, not neglect |
| Frontend | React + Tailwind + Recharts + D3.js | Recharts for charts; D3 for peer force-graph |
| Containerization | Docker Compose (whole stack) | One-command demo setup |

# 7. SOTA COMPARISON

Every existing tool has a structural gap that VaultScope fills. This table is the competitive moat — reference it during judge Q&A.

| Tool | What It Does | Critical Gap vs VaultScope |
| --- | --- | --- |
| ike-scan | Sends IKE packets, records responses, fingerprints via retransmission timing | Active only. No passive mode. No established-session analysis. No ESP/AH. Last commit 2012. |
| ikeforce | Aggressive mode group-name brute force + transform enumeration | Python 2, unmaintained, no Python 3 port. Single-function. Zero analysis or reporting. |
| iker.py | ike-scan wrapper, automates transform enum, basic text report | Still purely active. No live capture. No ESP. No session continuity. No scoring. |
| IKEv3Analytica | Modern Python IKEv1+IKEv2 scanner | Scan-based (point at IP, probe). No passive. No pcap. No graph. No anomaly detection. |
| Wireshark | Manual IKE/ESP packet decode | Expert tool for humans. No automation, no scoring, no remediation, no reporting. |
| Tufin / AlgoSec | Firewall policy management + compliance | Analyses config files, not wire traffic. Zero visibility into runtime cipher negotiation. |
| Cisco SecureX / Stealthwatch | Flow-level anomaly detection | Sees that IPsec traffic exists; does not decode IKE at all. No cipher-level analysis. |
| Nessus IPsec plugins | Active probes for known vulns | Active only. No session continuity. No graph. No ESP classification. |
| VaultScope (ours) | Passive capture → full IKE decode → ESP traffic classification → graph → remediation | First tool combining all: passive mode, traffic-type ML, vendor-specific config diffs, SIEM-ready export, peer graph with risk propagation. |

# 8. TASK ASSIGNMENT — 4 ENGINEERS

Tasks are designed for async work. Each person owns a vertical slice. Dependencies between slices are communicated via the shared data model (Section 5) and API contract (Section 11). No engineer needs another's feature to be complete to start working — they work against the shared schema and mock data until integration.

## P1 — Testbed, Dataset & Ingestion

Owns: Stage 0 (testbed), Stage 1 (ingestion engine), dataset deliverable.

| Task ID | Task | Output / Done Criteria |
| --- | --- | --- |
| P1-T1 | Write Docker Compose + strongSwan config generator script (Cartesian product of all dimensions) | docker-compose up brings up all peer pairs; configs auto-generated |
| P1-T2 | Wire up traffic generators (sipp, ffmpeg, curl, swaks, hping3, burst-gen) | Each traffic type flows through its tunnel for 120s; confirmed by tshark filter |
| P1-T3 | Write capture script: tcpdump per tunnel → named pcap + ground-truth JSON | data/ directory populated; naming convention enforced; JSON schema validates |
| P1-T4 | Generate full dataset (~50 pcaps per traffic class per cipher group; target ≥300 total) | Dataset complete; balance checked across all 6 traffic classes |
| P1-T5 | Build ingestion engine: pyshark wrapper + scapy fallback + session bucketing | Given any pcap, returns dict of (session_id → [IKE packets, ESP packets]) |
| P1-T6 | Handle edge cases: mid-session capture, NAT-T (UDP 4500), fragmented IKE | Edge-case pcap fixtures; all parse without crash; flags set correctly |
| P1-T7 | Expose ingestion as Python module + FastAPI endpoint (POST /ingest) | curl POST with pcap file → 200 with session count; integrates with P3 backend |

## P2 — IKE Parser, Flow Features & ML Classifiers

Owns: Stage 2 (IKE parser), Stage 3 (flow feature extractor), Stage 4a (protocol classifier), Stage 4b (traffic-type classifier).

| Task ID | Task | Output / Done Criteria |
| --- | --- | --- |
| P2-T1 | Build IKEv2 parser: SA_INIT + IKE_AUTH → full VPNSession object | Given IKEv2 pcap, returns correct encryption/dh_group/pfs_status/auth_method |
| P2-T2 | Build IKEv1 parser: Main Mode + Aggressive Mode detection | Aggressive Mode: ID payload in message 1 detected; aggressive_mode=true set |
| P2-T3 | Handle IKEv1 Phase 2 (Quick Mode): PFS detection, phase 2 cipher | PFS correctly set from KE payload presence; phase 2 encryption extracted |
| P2-T4 | Handle edge cases: mid-session (pfs_status=unknown), fragmented IKE flag, VID fingerprinting | All flags set correctly; vendor detected from VID DB on strongSwan/Cisco/Juniper samples |
| P2-T5 | Build ESP flow feature extractor: 12-feature vector per flow | Given ESP-only pcap, returns feature vector dict; all 12 features populated |
| P2-T6 | Validate burst_gap_ratio + payload_size_var_burst features separate Video from Web | Scatter plot or quick RF shows clear Video/Web separation in feature space |
| P2-T7 | Train traffic-type classifier (RF baseline): load dataset, extract features, train, evaluate | Confusion matrix generated; per-class F1 reported; model.pkl saved |
| P2-T8 | Train XGBoost variant; compare vs RF; pick best on F1-macro | Model selection documented; winning model saved + versioned |
| P2-T9 | Build protocol/crypto classifier for ambiguous partial captures | Given truncated IKE pcap, outputs VPNSession with confidence_source=classifier |
| P2-T10 | Expose classifiers as Python module; predict() accepts VPNSession + flow features | Unit tests pass; integrates with P3 backend pipeline |

## P3 — Rule Engine, Scoring, Reports & Backend API

Owns: Stage 4c (rule engine), Stage 5 (scoring + reports), FastAPI backend, DB, API contract.

| Task ID | Task | Output / Done Criteria |
| --- | --- | --- |
| P3-T1 | Implement rule engine: load rules.yaml, evaluate VPNSession, return triggered rules + severities | All 15 rules from table evaluated correctly on fixture sessions |
| P3-T2 | Implement scoring model: weighted penalty → composite score → overall_severity | CRITICAL rule auto-sets severity regardless of composite score |
| P3-T3 | Implement vendor remediation engine: fingerprint → config template lookup → diff string | All 5 vendors + generic produce correct config diffs for a CRITICAL DH-group-2 finding |
| P3-T4 | Build Jinja2 executive report template (PDF via WeasyPrint) | Given 3 sessions with findings, produces 1-2 page PDF with score + top 3 findings |
| P3-T5 | Build Jinja2 technical report template (HTML + PDF) | Full session table, per-finding CVE links, vendor config diffs, classifier confusion matrix embedded |
| P3-T6 | Build JSON + CEF export | JSON matches data model schema (Section 5); CEF format validates against CEF spec |
| P3-T7 | Build FastAPI app: routes POST /ingest, GET /sessions, GET /session/{id}, POST /report | All routes return correct schemas; Swagger UI auto-generated |
| P3-T8 | Build WebSocket endpoint /ws/live for real-time session stream | WebSocket sends new session event within 1s of IKE handshake completion in live mode |
| P3-T9 | Wire full backend pipeline: ingestion → parser → features → classifiers → rules → scoring → DB | POST /ingest with test pcap → GET /sessions returns fully populated session records |
| P3-T10 | Anomaly event detection + /events endpoint | Transform brute-force, aggressive mode probe, SPI collision anomalies detected on fixture pcaps |

## P4 — Frontend Dashboard & Integration

Owns: Stage 6 (React UI), Docker Compose integration, demo video.

| Task ID | Task | Output / Done Criteria |
| --- | --- | --- |
| P4-T1 | Scaffold React app: Tailwind, Recharts, D3, React Router; connect to FastAPI base URL | App runs; health check shows backend connected |
| P4-T2 | Build upload view: pcap file drop → POST /ingest → loading state → redirect to analysis | File drop works; progress indicator shown; redirects on success |
| P4-T3 | Build session table: sortable by risk score / cipher / IP; filterable; paginated | Sort and filter work on ≥100 mock rows; CRITICAL rows highlighted red |
| P4-T4 | Build session drilldown panel: full IKE decode, rule violations, classifier confidence, vendor config diff | Clicking any session shows all fields from data model; config diff shown in monospace block |
| P4-T5 | Build D3 peer force-graph: nodes = peers, edges = sessions, colour = worst-session risk | Graph renders; drag nodes; click node = filter table to that peer; click edge = open session |
| P4-T6 | Build aggregate dashboard: risk histogram, traffic-type pie chart, threat matrix heatmap | All 3 charts render from real session data |
| P4-T7 | Wire WebSocket: /ws/live → live session table updates without page reload | New sessions appear in table within 2s of backend detection in live capture mode |
| P4-T8 | Build export panel: executive PDF, technical HTML, JSON/CEF download buttons | All 4 downloads work; files are non-empty and valid |
| P4-T9 | Historical diff view: load two pcaps, show added/removed peers, degraded sessions | Diff correctly shows changed sessions between two test pcaps |
| P4-T10 | Docker Compose integration: frontend container + nginx; docker-compose up → full stack works | Single docker-compose up brings full stack; localhost:3000 loads working UI |

# 9. COMMIT MAP & TEST CASES

Every commit corresponds to a task ID. Each task has a mandatory test that must pass before merging. Tests live in tests/ mirroring the core/ structure. All tests are runnable with: pytest tests/ from repo root.

## P1 Commits & Tests

### P1-T1: Testbed config generator

commit: feat(testbed): cartesian config generator for strongSwan peer pairs

test file: tests/testbed/test_config_generator.py

def test_all_cipher_configs_generated():

configs = generate_configs()

assert len(configs) >= 96  # 2×6×3×2 minimum without traffic-type dim

def test_config_naming_convention():

configs = generate_configs()

for c in configs:

assert re.match(r"^(tunnel|transport)_.+_.+_(pfs|nopfs)_(ipv4|ipv6)$", c.name)

def test_strongswan_config_syntax_valid():

# run swanctl --load-conns --file <generated> and check return code 0

config = generate_configs()[0]

result = subprocess.run(["swanctl", "--load-conns", "--file", config.path])

assert result.returncode == 0

### P1-T3: Capture script + dataset schema

commit: feat(dataset): pcap capture script + ground-truth JSON schema

test file: tests/testbed/test_dataset_schema.py

def test_label_json_schema_valid():

for label_file in Path("data/labels").glob("*.json"):

label = json.load(open(label_file))

assert all(k in label for k in [

"filename","ipsec_mode","ike_version","encryption",

"dh_group","pfs_enabled","ip_version","traffic_type","known_weak"

])

def test_pcap_exists_for_every_label():

for label_file in Path("data/labels").glob("*.json"):

label = json.load(open(label_file))

pcap = Path("data/pcaps") / label["filename"]

assert pcap.exists() and pcap.stat().st_size > 0

def test_dataset_class_balance():

counts = Counter(json.load(open(f))["traffic_type"]

for f in Path("data/labels").glob("*.json"))

for cls in ["VoIP","Video","Web","Email","ICMP","Chat"]:

assert counts[cls] >= 30, f"{cls} underrepresented: {counts[cls]}"

### P1-T5: Ingestion engine

commit: feat(ingestion): pyshark wrapper + scapy fallback + session bucketing

test file: tests/ingestion/test_ingestion.py

def test_ikev2_sessions_bucketed(ikev2_pcap_fixture):

sessions = ingest(ikev2_pcap_fixture)

assert len(sessions) >= 1

for sid, pkts in sessions.items():

assert "ike_packets" in pkts and "esp_packets" in pkts

def test_ikev1_sessions_bucketed(ikev1_pcap_fixture):

sessions = ingest(ikev1_pcap_fixture)

assert len(sessions) >= 1

def test_nat_traversal_port_4500(nat_t_pcap_fixture):

sessions = ingest(nat_t_pcap_fixture)

assert any(s["nat_traversal"] for s in sessions.values())

def test_mid_session_capture_flagged(esp_only_pcap_fixture):

sessions = ingest(esp_only_pcap_fixture)

assert all(not s["capture_complete"] for s in sessions.values())

def test_fragmented_ike_flag(fragmented_ike_pcap_fixture):

sessions = ingest(fragmented_ike_pcap_fixture)

assert any(s["fragmented_ike"] for s in sessions.values())

## P2 Commits & Tests

### P2-T1: IKEv2 parser

commit: feat(ike_parser): IKEv2 SA_INIT + IKE_AUTH full VPNSession extraction

test file: tests/ike_parser/test_ikev2_parser.py

@pytest.mark.parametrize("cipher,expected_enc,expected_dh", [

("aes256gcm_ecp521_pfs", "AES-256-GCM", "ECP521"),

("aes128cbc_sha256_modp2048", "AES-128-CBC", "MODP2048"),

("3des_sha1_modp1024", "3DES-CBC", "MODP1024"),

])

def test_ikev2_cipher_extraction(cipher, expected_enc, expected_dh, fixture_pcap):

session = parse_ikev2(fixture_pcap(cipher))

assert session.encryption == expected_enc

assert session.dh_group == expected_dh

def test_ikev2_pfs_enabled(pfs_pcap_fixture):

session = parse_ikev2(pfs_pcap_fixture)

assert session.pfs_status == "enabled"

def test_ikev2_pfs_disabled(no_pfs_pcap_fixture):

session = parse_ikev2(no_pfs_pcap_fixture)

assert session.pfs_status == "disabled"

def test_ikev2_transport_mode(transport_pcap_fixture):

session = parse_ikev2(transport_pcap_fixture)

assert session.mode == "transport"

def test_mid_session_pfs_unknown(esp_only_fixture):

session = parse_ikev2(esp_only_fixture)

assert session.pfs_status == "unknown"  # NOT "disabled"

### P2-T2: IKEv1 + Aggressive Mode

commit: feat(ike_parser): IKEv1 Main Mode + Aggressive Mode detection

test file: tests/ike_parser/test_ikev1_parser.py

def test_aggressive_mode_detected_by_id_payload_in_message_1(aggr_pcap):

session = parse_ikev1(aggr_pcap)

assert session.aggressive_mode == True

# ID payload present in first IKE message — not inferred from count

def test_main_mode_no_aggressive_flag(main_mode_pcap):

session = parse_ikev1(main_mode_pcap)

assert session.aggressive_mode == False

def test_ikev1_retransmission_does_not_flip_mode(retransmit_pcap):

# retransmission changes message count — should not affect mode detection

session = parse_ikev1(retransmit_pcap)

assert session.aggressive_mode == False  # main mode pcap with retransmit

### P2-T5 + P2-T6: Flow feature extractor

commit: feat(flow): ESP flow feature extractor + Video/Web separation validation

test file: tests/flow/test_feature_extractor.py

def test_all_12_features_populated(esp_pcap_fixture):

features = extract_flow_features(esp_pcap_fixture)

required = ["pkt_size_mean","pkt_size_std","pkt_size_p10","pkt_size_p90",

"iat_mean_ms","iat_std_ms","dir_ratio","burst_count",

"burst_gap_ratio","payload_size_var_burst","flow_duration_sec",

"pkt_total","rate_pps"]

assert all(k in features for k in required)

assert all(isinstance(features[k], (int,float)) for k in required)

def test_voip_pkt_size_mean_under_300(voip_esp_fixture):

f = extract_flow_features(voip_esp_fixture)

assert f["pkt_size_mean"] < 300

assert f["iat_std_ms"] < 5  # regular timing

def test_video_burst_gap_ratio_higher_than_web(video_esp, web_esp):

fv = extract_flow_features(video_esp)

fw = extract_flow_features(web_esp)

assert fv["burst_gap_ratio"] > fw["burst_gap_ratio"]

def test_icmp_pkt_total_low_rate_pps_low(icmp_esp_fixture):

f = extract_flow_features(icmp_esp_fixture)

assert f["rate_pps"] < 5

### P2-T7 + P2-T8: Traffic-type classifier

commit: feat(classifiers): train + evaluate RF and XGBoost traffic-type classifiers

test file: tests/classifiers/test_traffic_classifier.py

def test_classifier_predicts_all_6_classes():

model = load_model("models/traffic_classifier.pkl")

for cls in ["VoIP","Video","Web","Email","ICMP","Chat"]:

sample = fixture_feature_vector(cls)

pred, conf = model.predict(sample)

# must predict the right class for clear-signal fixtures

assert pred == cls

assert conf >= 0.6

def test_voip_high_confidence(voip_feature_fixture):

model = load_model("models/traffic_classifier.pkl")

pred, conf = model.predict(voip_feature_fixture)

assert pred == "VoIP" and conf >= 0.85

def test_f1_macro_above_threshold():

# loaded from eval artifacts generated at training time

metrics = json.load(open("models/eval_metrics.json"))

assert metrics["f1_macro"] >= 0.70  # honest threshold, not overclaimed

def test_confusion_matrix_artifact_exists():

assert Path("models/confusion_matrix.json").exists()

## P3 Commits & Tests

### P3-T1 + P3-T2: Rule engine + scoring

commit: feat(rules): rule engine + weighted scoring model

test file: tests/rules/test_rule_engine.py

def test_des_encryption_triggers_r01_critical():

session = make_session(encryption="DES-CBC")

result = evaluate_rules(session)

assert "R01" in result.triggered_rules

assert result.overall_severity == "CRITICAL"

def test_dh_group2_triggers_r04_critical():

session = make_session(dh_group="MODP1024")

result = evaluate_rules(session)

assert "R04" in result.triggered_rules

assert result.overall_severity == "CRITICAL"

def test_critical_rule_overrides_composite_score():

# Even with only one violation, CRITICAL rule must set severity=CRITICAL

session = make_session(encryption="AES-256-GCM", dh_group="MODP1024")

result = evaluate_rules(session)

assert result.overall_severity == "CRITICAL"

def test_pfs_unknown_not_penalised():

session = make_session(pfs_status="unknown")

result = evaluate_rules(session)

assert "R10" not in result.triggered_rules  # no penalty for unknown

def test_clean_session_scores_100():

session = make_session(

ike_version="IKEv2", encryption="AES-256-GCM", dh_group="ECP521",

pfs_status="enabled", auth_method="RSA", sa_lifetime_sec=3600

)

result = evaluate_rules(session)

assert result.risk_score == 100

assert result.overall_severity == "SAFE"

### P3-T3: Vendor remediation engine

commit: feat(remediation): vendor-specific config diff generator

test file: tests/rules/test_remediation.py

@pytest.mark.parametrize("vendor,expected_keyword", [

("Cisco ASA",   "crypto ikev2 policy"),

("strongSwan",  "proposals ="),

("Juniper SRX", "set security ike proposal"),

("Fortinet",    "config vpn ipsec"),

("Palo Alto",   "ike-crypto-profile"),

("unknown",     "RFC 8247"),

])

def test_vendor_remediation_contains_correct_syntax(vendor, expected_keyword):

finding = make_finding(rule_id="R04", vendor=vendor)

diff = generate_remediation(finding)

assert expected_keyword in diff

def test_remediation_specifies_strong_dh_group():

finding = make_finding(rule_id="R04", vendor="strongSwan")

diff = generate_remediation(finding)

# must recommend group 19 or higher, not group 14

assert "ecp256" in diff.lower() or "ecp521" in diff.lower()

### P3-T7 + P3-T9: FastAPI pipeline

commit: feat(api): FastAPI routes + full pipeline wiring

test file: tests/api/test_api.py

def test_post_ingest_returns_session_count(client, test_pcap):

resp = client.post("/ingest", files={"file": open(test_pcap,"rb")})

assert resp.status_code == 200

assert resp.json()["session_count"] >= 1

def test_get_sessions_returns_list(client):

resp = client.get("/sessions")

assert resp.status_code == 200

assert isinstance(resp.json(), list)

def test_session_record_fully_populated(client, test_pcap):

client.post("/ingest", files={"file": open(test_pcap,"rb")})

sessions = client.get("/sessions").json()

s = sessions[0]

assert all(k in s for k in ["session_id","ike","traffic_prediction","security_assessment"])

assert s["ike"]["encryption"] is not None

assert 0 <= s["security_assessment"]["risk_score"] <= 100

def test_critical_session_has_findings(client, weak_pcap):

client.post("/ingest", files={"file": open(weak_pcap,"rb")})

sessions = client.get("/sessions").json()

critical = [s for s in sessions if s["security_assessment"]["overall_severity"]=="CRITICAL"]

assert len(critical) >= 1

assert len(critical[0]["security_assessment"]["findings"]) >= 1

assert critical[0]["security_assessment"]["findings"][0]["remediation"] != ""

## P4 Commits & Tests

### P4-T2: Upload flow

commit: feat(frontend): pcap upload flow + loading state + redirect

test: Cypress e2e  tests/e2e/test_upload.cy.js

it("uploads pcap and redirects to session table", () => {

cy.visit("/")

cy.get("[data-testid=pcap-drop]").attachFile("fixtures/test_weak.pcap")

cy.get("[data-testid=upload-progress]").should("be.visible")

cy.url({ timeout: 15000 }).should("include", "/sessions")

cy.get("[data-testid=session-row]").should("have.length.gte", 1)

})

### P4-T3: Session table

commit: feat(frontend): session table with sort, filter, risk highlight

test: Cypress  tests/e2e/test_session_table.cy.js

it("sorts by risk score descending", () => {

cy.get("[data-testid=sort-risk]").click()

cy.get("[data-testid=session-row]").first()

.should("have.attr","data-severity","CRITICAL")

})

it("highlights CRITICAL rows red", () => {

cy.get("[data-severity=CRITICAL]").should("have.css","background-color")

.and("match", /rgb\(24[0-9]|25[0-5]/)  // red range

})

it("filters by IKE version", () => {

cy.get("[data-testid=filter-ike-version]").select("IKEv1")

cy.get("[data-testid=session-row]").each(row => {

cy.wrap(row).find("[data-field=ike_version]").should("contain","IKEv1")

})

})

### P4-T5: Peer graph

commit: feat(frontend): D3 peer graph with risk colouring + session drilldown

test: Cypress  tests/e2e/test_peer_graph.cy.js

it("renders a node per unique peer IP", () => {

const expectedPeers = 4  // from fixture session data

cy.get("[data-testid=graph-node]").should("have.length", expectedPeers)

})

it("CRITICAL node has red fill", () => {

cy.get("[data-testid=graph-node][data-severity=CRITICAL]")

.invoke("attr","fill").should("match", /^#[Ff][0-9a-fA-F]{5}$|red/)

})

it("clicking node filters session table to that peer", () => {

cy.get("[data-testid=graph-node]").first().click()

cy.get("[data-testid=session-row]").each(row => {

cy.wrap(row).find("[data-field=initiator_ip],[data-field=responder_ip]")

.should("contain", "192.168")  // fixture peer IP

})

})

### P4-T7: WebSocket live update

commit: feat(frontend): WebSocket session stream + live table update

test: Cypress  tests/e2e/test_live_mode.cy.js

it("new session appears in table within 2 seconds of WS event", () => {

cy.visit("/live")

cy.window().then(win => {

win.mockWS.send(JSON.stringify(fixtures.newSession))

})

cy.get("[data-testid=session-row][data-id=test-session-99]", { timeout: 2000 })

.should("be.visible")

})

# 10. DATASET SPECIFICATION

The labeled dataset is an explicit deliverable from the PS. Target: ≥300 pcap files total, balanced across traffic classes.

| Traffic Class | Generator | Expected Flow Signature | Target Samples |
| --- | --- | --- | --- |
| VoIP | sipp (SIP/RTP) | Small (~180B), constant-rate, regular IAT, dir_ratio≈1.0 | ≥50 per cipher group |
| Video | ffmpeg RTP stream | Large (~1400B), bursty, high burst_gap_ratio, high throughput | ≥50 |
| Web | curl + headless Chromium | Variable size, bursty then idle, high dir_ratio (down-heavy) | ≥50 |
| Email | Postfix + swaks | Small bursts, high dir_ratio (up on send), then idle | ≥50 |
| ICMP | ping / hping3 | Tiny, fixed-size, sparse, low rate_pps | ≥50 |
| Chat (WhatsApp proxy) | Custom Python burst gen | Small, bursty, irregular, bidirectional. NOTE: approximation — disclosed in reports | ≥50 |

## Known Limitations — Disclosed in Technical Report

- WhatsApp traffic cannot be literally reproduced in an isolated lab (requires live Meta servers). Substitute: Python generator approximating its packet-size/timing signature. Explicitly disclosed.

- Dataset is lab-controlled: clean network, no cross-traffic noise. Real-world accuracy will be lower than lab evaluation metrics. Reported with confusion matrix, not hidden.

- Mid-session captures (no IKE observed): ML classifier operates on flow features only; IKE-derived features are null. Confidence score reflects this.

# 11. API CONTRACT

FastAPI auto-generates Swagger UI at /docs. These are the core routes. All responses are JSON.

| Method | Route | Request | Response | Owner |
| --- | --- | --- | --- | --- |
| POST | /ingest | multipart/form-data: file=<pcap> | { session_count: int, job_id: str } | P1 |
| GET | /sessions | query: ?severity=CRITICAL&limit=50&offset=0 | Array of VPNSession (full schema) | P3 |
| GET | /session/{id} | path: session_id | Full VPNSession object | P3 |
| GET | /sessions/diff | query: ?base_job=<id>&compare_job=<id> | { added: [], removed: [], degraded: [] } | P3 |
| POST | /report/{job_id} | body: { type: "executive"¦"technical"¦"json"¦"cef" } | { download_url: str } | P3 |
| GET | /events | query: ?session_id=<id>&severity=HIGH | Array of AnomalyEvent | P3 |
| GET | /health | none | { status: "ok", version: str } | P3 |
| WS | /ws/live | live NIC name in query param | Stream of VPNSession events as JSON | P3 |

## Mock Data Contract (for P4 to start async)

P4 works against this mock server (JSON fixture files served by a simple Express/Python static server) before the real backend is ready:

GET /sessions → data/mock/sessions.json     (10 sessions, mix of severities)

GET /session/sess-001 → data/mock/session_001.json

WS /ws/live → data/mock/ws_stream.json      (10 session events, 1 per second)

# 12. RISK REGISTER

| Risk | Likelihood | Impact | Mitigation |
| --- | --- | --- | --- |
| nfstream fails to build in Docker | HIGH | MEDIUM | Pre-test Docker image before event. Custom scapy aggregator is primary path — nfstream is optional accelerator only. |
| Video vs Web classifier confusion | HIGH | MEDIUM | burst_gap_ratio + payload_size_var_burst features specifically added for this. Report confusion matrix honestly — F1≥0.70 is acceptable threshold. |
| IKEv1 fragmentation miscount alters mode detection | MEDIUM | HIGH | Aggressive Mode detected by ID payload presence in message 1, not message count. Covered in P2-T2 test. |
| mid-session capture falsely flags PFS=disabled | HIGH | HIGH | pfs_status=unknown for incomplete captures. Rule R10 does not fire on unknown. Covered in P2-T1 test. |
| WeasyPrint PDF rendering breaks in Docker | MEDIUM | MEDIUM | Pre-test in Docker. Fallback: ReportLab. Both in requirements.txt. |
| Dataset imbalance between cipher groups | MEDIUM | MEDIUM | test_dataset_class_balance() enforces ≥30 per class before ML training begins. |
| WhatsApp proxy too different from real traffic | HIGH | LOW | Disclosed explicitly in dataset spec, technical report, and judge Q&A. Not hidden. |
| P4 blocked waiting for backend | HIGH | HIGH | Mock data contract + fixture JSON server lets P4 start day 1 with full UI development. |

# 13. DELIVERABLES CHECKLIST

All items required for submission. Track status in the repo README.

| # | Deliverable | Owner | Done When |
| --- | --- | --- | --- |
| 1 | Labeled dataset (pcaps + JSONs, ≥300 files) | P1 | data/ directory zippable; test_dataset_class_balance passes |
| 2 | Trained AI model artifacts (model.pkl + eval_metrics.json + confusion_matrix.json) | P2 | models/ directory exists; test_f1_macro_above_threshold passes |
| 3 | Working prototype (docker-compose up → full stack) | P4 | Single command brings up stack; POST /ingest with demo pcap returns sessions |
| 4 | Executive PDF report (from demo pcap) | P3 | PDF is non-empty; contains overall score and ≥1 finding in plain English |
| 5 | Technical HTML report (from demo pcap) | P3 | HTML contains full session table, CVE links, vendor config diffs |
| 6 | JSON/CEF export | P3 | JSON validates against schema (Section 5); CEF parses correctly |
| 7 | Dashboard demo (live in browser) | P4 | Peer graph renders; session drilldown works; export buttons produce files |
| 8 | Demo video (3–5 min) | All | Screen recording: upload → sessions → graph → drilldown → report download |
| 9 | This document (full product spec) | All | Present at submission |
| 10 | API reference (/docs Swagger) | P3 | Auto-generated; all routes documented with request/response schemas |
| 11 | Setup guide (README.md) | P4 | Single docker-compose up instruction; prerequisites listed |
| 12 | Disclosed limitations section | All | WhatsApp proxy, lab vs real-world accuracy, mid-session captures — all documented |

VaultScope — SIH26160 · NTRO · SIH 2026